| Contractor: Inspector: | Date: Time: |  |  | Consent #: |  |  | Site: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Site Inspection of Erosion and Sediment Control Practices |  |  |  |  |  |  |  |
| Erosion and Sediment Control Practice |  | Yes | No |  | N/A | Corrective Action |  |
| General Information |  |  |  |  |  |  |  |
| Do you know what receiving system the project drains into |  |  |  |  |  |  |  |
| Are you aware of local rainfall patterns during various times of the year |  |  |  |  |  |  |  |
| Soil types and erosion potential for site |  |  |  |  |  |  |  |
| Is a copy of the erosion and sediment control plan on site |  |  |  |  |  |  |  |
| Is temporary fencing placed in areas where no construction is to take place |  |  |  |  |  |  |  |
| Construction |  |  |  |  |  |  |  |
| Area cleared of unsuitable material and smooth graded |  |  |  |  |  |  |  |
| Geotextile placed over this area ensuring it is properly pinned and overlapped |  |  |  |  |  |  |  |
| At least 10 m of aggregate, 4 m wide and 1.5 x aggregate size in depth. Aggregate 100-105 mm washed aggregate |  |  |  |  |  |  |  |
| Ensure that vehicles cannot bypass the entranceway |  |  |  |  |  |  |  |
| Can be used with a shaker ramp (cattle stops) or with a wheel wash |  |  |  |  |  |  |  |
| Maintenance |  |  |  |  |  |  |  |
| Inspect weekly and after each rain |  |  |  |  |  |  |  |
| Maintain the entranceway to prevent it becoming a source of sediment |  |  |  |  |  |  |  |
| If used with a wheel wash, ensure that this drains to an approved sediment retention practice. |  |  |  |  |  |  |  |
| Decommissioning |  |  |  |  |  |  |  |
| Remove aggregate and geotextile and stabilise. At this point ensure that traffic is kept off of the area |  |  |  |  |  |  |  |